A General Optimization-based Framework for Local Odometry

Estimation with Multiple Sensors

Tong Qin, Jie Pan, Shaozu Cao, and Shaojie Shen

# arXiv: 1901.03638v1  [cs.CV]  11 Jan 2019

Abstract— Nowadays, more and more sensors are equipped on  robots to increase  robustness  and  autonomous  ability. We have seen various sensor suites equipped on different platforms, such as stereo cameras on ground vehicles, a monocular camera with an IMU  (Inertial Measurement Unit) on mobile phones, and  stereo  cameras  with  an  IMU  on  aerial  robots. Although many algorithms for state estimation have been proposed in the past, they are usually applied to a single sensor or a speciﬁc sensor suite. Few of them can be employed with multiple sensor choices. In this paper, we proposed a general optimization-based framework for odometry estimation, which supports multiple sensor sets. Every sensor is treated as a general factor in our framework.  Factors  which  share  common  state  variables  are summed together to build the optimization problem. We further demonstrate  the  generality  with  visual  and  inertial  sensors, which form three sensor suites  (stereo cameras, a monocular camera with an IMU, and stereo cameras with an IMU). We validate the performance of our system on public datasets and through real-world experiments with multiple sensors. Results are  compared  against  other  state-of-the-art  algorithms.  We highlight that our system is a general framework, which can easily fuse various sensors in a pose graph optimization. Our implementations are open source1.

I.  INTRODUCTION

Real-time  6-DoF  (Degrees  of Freedom)  state estimation is  a  fundamental  technology  for  robotics.  Accurate  state estimation  plays  an  important  role  in  various  intelligent applications, such as robot exploration, autonomous driving, VR (Virtual Reality) and AR (Augmented Reality). The most common sensors we use in these applications are cameras. A large number of impressive vision-based algorithms for pose estimation has been proposed over the last decades, such as [1]–[5]. Besides cameras, the IMU is another popular option for state estimation. The IMU can measure acceleration and angular velocity at a high frequency, which is necessary for low-latency pose feedback in real-time applications. Hence, there are numerous research works fusing vision and IMU together, such as  [6]–[12]. Another popular sensor used in state  estimation  is  LiDAR.  LiDAR-based  approaches  [13] achieve  accurate pose estimation in  a conﬁned local envi- ronment. Although a lot of algorithms have been proposed in the past, they are usually applied to a single input sensor or a speciﬁc sensor suite.

Recently, we have seen platforms equipped with various sensor  sets,  such  as  stereo  cameras  on  ground  vehicles,  a monocular camera with an IMU on mobile phones,  stereo

All     authors     are     with     the     Department      of     Electronic     and Computer    Engineering,    Hong    Kong    University    of    Science     and Technology,     Hong     Kong,     China.      {tong .qin,  jie .pan, shaozu.cao}@connect .ust .hk,  eeshaojie@ust .hk.

1https://github.com/HKUST-Aerial-Robotics/VINS-Fusion

Fig.  1.     An  illustration  of the  proposed  framework  for  state estimation, which supports multiple sensor choices, such as stereo cameras, a monocular camera  with  an  IMU,  and  stereo  cameras  with  an  IMU.  Each  sensor  is treated as a general factor. Factors which share common state variables are summed together to build the optimization problem.

cameras with  an IMU  on  aerial robots. However,  as most traditional algorithms were designed for a single sensor or a  speciﬁc   sensor  set,  they  cannot  be  ported  to  different platforms. Even for  one platform, we need to  choose  dif- ferent sensor combinations in different scenarios. Therefore, a  general  algorithm  which  supports  different  sensor  suites is  required.  Another  practical  requirement  is  that  in  case of  sensor  failure,  an  inactive  sensor  should  be  removed and an alternative  sensor  should be added into the  system quickly. Hence, a general algorithm which is compatible with multiple sensors is in need.

In  this  paper,  we  propose  a  general  optimization-based framework  for  pose   estimation,  which  supports  multiple sensor combinations. We further demonstrate it with visual and inertial sensors, which form three sensor suites (stereo cameras, a monocular camera with an IMU, and stereo cam- eras with an IMU). We can easily switch between different sensor combinations. We highlight the contribution of this paper as follows:

•   a  general optimization-based framework for state esti- mation, which supports multiple sensors.

•   a detailed demonstration of state estimation with visual and inertial sensors, which form different sensor suites (stereo cameras,  a monocular camera +  an IMU,  and stereo cameras + an IMU).

•   an  evaluation  of the  proposed  system  on  both public datasets and real experiments.

# •   open-source code for the community.

II.  RELATED WORK

State estimation has been a popular research topic over the last decades. A large number of algorithms focus on accurate 6-DoF  pose   estimation.  We   have  seen  many  impressive approaches  that  work  with  one  kind  of  sensor,   such  as visual-based methods  [1]–[5],  LiDAR-based methods  [13], RGB-D based methods [14]. and event-based methods [15]. Approaches work with a monocular camera is hard to achieve 6-DoF pose estimation,  since  absolute  scale cannot be re- covered from a single camera. To increase the observability and robustness, multiple sensors which have complementary properties are fused together.

There are two trends of approaches for multi-sensor fu- sion. One is ﬁlter-based methods, the other is optimization- based methods. Filter-based methods are usually achieved by EKF (Extended Kalman Filter). Visual and inertial measure- ments are usually ﬁltered together for 6-DoF state estimation. A high-rate inertial sensor is used for state propagation and visual  measurements  are  used  for  the  update  in   [9,  16]. MSCKF [6, 7] was a popular EKF-based VIO (Visual Inertial Odometry),  which  maintained  several  camera  poses   and leveraged multiple camera views to form the multi-constraint update. Filter-based methods usually linearize states earlier and  suffer  from  error induced by  inaccurate  linear points. To  overcome the inconsistency  caused by linearized  error,

# observability constrained EKF [17] was proposed to improve

accuracy  and  consistency.  An  UKF  (Unscented  Kalman Filter)  algorithm  was  proposed  in  [18],  where  visual,  Li- DAR and GPS measurements were fused together. UKF is an  externsion  of  EKF  without  analytic  Jacobians.  Filter- based  methods  are  sensitive  to  time  synchronization.  Any late-coming  measurements  will  cause  trouble  since  states cannot be propagated back in ﬁlter procedure. Hence, spe- cial  ordering mechanism is required to make  sure that  all measurements from multiple sensors are in order.

Optimization-based  methods  maintain  a  lot  of measure- ments and optimize multiple variables at once, which is also known as Bundle Adjustment  (BA). Compared with ﬁlter- based  method,  optimization-based  method  have  advantage in time synchronization. Because the big bundle serves as a nature buffer, it can easily handle the case when measurments from multiple sensors come in disorder. Optimization-based algorithms  also  outperform  the  ﬁlter-based  algorithms  in term  of accuracy  at the cost  of computational complexity. Early  optimization  solvers,  such  as  G2O   [19],  leveraged the  Gauss-Newton   and  Levenberg-Marquardt   approaches to  solve  the  problem.  Although  the  sparse  structure  was employed  in  optimization  solvers,  the  complexity  grown quadratically with the number of states and measurements. In order to achieve real-time performance, some algorithms have  explored  incremental  solvers,  while  others  bounded the  size  of  the  pose  graph.  iSAM2  [20]  was  an  efﬁcient incremental solver, which reused the previous optimization result to reduce computation when new measurements came. The optimization iteration only updated a small part of states

Fig. 2.      A  graphic  illustration  of the pose  graph. Each node represents states (position, orientation, velocity and so on) at one moment. Each edge represents a factor, which is derived by one measurement. Edges constrain one state, two states or multiple states.

instead of the whole pose graph. Afterward, an accelerated solver was proposed in [21], which improved efﬁciency by reconstructing dense structure into sparse blocks. Methods, that  keep  a  ﬁxed  sized  of  pose  graph,  are  called  sliding- window approaches. Impressive optimization-based VIO ap- proaches,  such  as  [8,   10,   12],  optimized  variables  over a  bounded-size  sliding  window.  The  previous  states  were marginalized  into  a  prior  factor  without  loss  of  informa- tion  in  [8,  12].  In  this  paper,  we  adopt  a  sliding-window optimization-based framework for state estimation.

III.  SYSTEM OVERVIEW

The structure of proposed framework is shown in Fig. 1. Multiple  kinds  of  sensors  can  be  freely  combined.  The mesurement  of each  sensor  is  treated  as  a  general  factor. Factors  and  their  related  states  form  the  pose  graph.  An illustration  of pose  graph  is  shown  in  Fig. 2.  Each  node represents  states  (position, orientation, velocity and  so on) at  one  moment.  Each  edge  represents  a  factor,  which  is derived  by  one  measurement.  Factors  constrain  one  state, two states or multiple states. For IMU factor, it constrains two consecutive states by continuous motion restriction. For a visual landmark, its factor constrains multiple states since it is observed on multiple frames. Once the graph is built, optimizing it equals to ﬁnding the conﬁguration of nodes that match all edges as much as possible.

In this paper, we speciﬁcally demonstrate the system with visual  and inertial  sensors. Visual  and inertial  sensors can form three combinations for 6-DoF state estimation, which are stereo cameras, a monocular camera with an IMU, and stereo cameras with an IMU. A graphic illustration of the proposed framework with visual and inertial sensors is shown in  Fig. 3.  Several  camera  poses,  IMU  measurements  and visual measurements exist in the pose graph. The IMU and one of cameras are optional.

# IV.  METHODOLOGY

## A.  Problem Deﬁnition

1)  States:  Main  states that we need to estimate includes 3D position and orientation of robot’s center. In addition, we

Fig. 3.     A graphic illustration of the proposed framework with visual and inertial sensors. The IMU and one of cameras are optional. Therefore, it forms three types (stereo cameras, a monocular camera with an IMU, and stereo cameras with an IMU). Several camera poses, IMU measurements and visual measurements exist in the pose graph.

have other optional states, which are related to sensors. For cameras, depths or 3D locations of visual landmarks need to be estimated. For IMU, it produces another motion variable, velocity, Also, time-variant acceleration bias and gyroscope bias of the IMU are needed to be estimated. Hence, for visual and inertial  sensors, whole  states we need to estimate  are

## deﬁned as follows:

X = [p0, R0, p1, R1, ..., pn, Rn, xcam, ximu]

xcam  = [λ0 , λ 1, ..., λl ]                                                  (1)

ximu  = [v0, ba0, bg0, v1, ba1, bg1, ..., vn, ban, bgn ],

where p and R are basic system states, which correspond to position and orientation of body expressed in world frame. xcam  is camera-ralated state, which includes depth λ of each feature  observed  in  the  ﬁrst  frame.  ximu   is  IMU-ralated variable, which is composed of velocity v, acceleration bias ba  and gyroscope bias bg . ximu  can be omitted if we only use  stereo  camera  without  an  IMU.  The  translation  from sensors’ center to body’s center are assumed to be known, which are calibrated ofﬂine. In order to simplify the notation, we denote the IMU as body’s center (If the IMU is not used,

### we denote left camera as body’s center).

2)  Cost  Function:  The  nature  of  state  estimation  is  an MLE (Maximum Likelihood Estimation) problem. The MLE consists of the joint probability distribution of robot poses over a period of time. Under the assumption that all mea- surements are independent, the problem is typically derived as,

where  S  is  the   set  of  measurements,  which  come  from cameras,  IMU  and  other  sensors.  We  assume  the  uncer-

tainty of measurements is Gaussian distributed, p(zkt jX) ~

N(kt , Ωkt). Therefore, the negative log-likelihood of above-

mentioned equation is written as,

(3)

The  Mahalanobis  norm  is  deﬁned  as   ⅡrⅡ2Ω  =   rT Ω —1r.

h(·) is the sensor model, which is detailed in the following section. Then the state estimation is converted to a nonlinear least  squares  problem,  which  is  also  known  as  Bundle Adjustment (BA).

### B.  Sensor Factors

1)  Camera Factor:  The framework supports both monoc- ular and  stereo cameras. The intrinsic parameters of every camera and the extrinsic transformation between cameras are supposed to be known, which can be easily calibrated ofﬂine. For  each  camera  frame,  corner  features  [22]  are  detected. These features are tracked in previous frame by KLT tracker [23]. For the stereo setting, the tracker also matches features between the left image  and right image. According to the feature associations, we construct the camera factor with per feature in each frame. The camera factor is the reprojection process, which projects a feature from its ﬁrst observation into following frames.

Considering the feature l that is ﬁrst observed in the image i, the residual for the observation in the following image t is deﬁned as:

(4)

where  [uli, vli]  is  the  ﬁrst  observation  of the  l feature  that

appears in the i image. [ult, vlt] is the observation of the same

feature in the t image. πc  and πc—1  are the projection and

back-projection  functions  which  depend  on  camera  model

(pinhole,  omnidirectional  or  other  models).  T  is  the  4x4

homogeneous  transformation,  which  is     We  omit

some homogeneous terms for concise expression. Tcb  is the

extrinsic transformation from body center to camera center,

which  is  calibrated  ofﬂine.  The  covariance  matrix  Ωlt   of

reprojection  error  is  a  constant  value  in  pixel  coordinate,

which comes from the camera’s intrinsic calibration results.

This  factor  is  universal  for  both  left  camera  and  right

camera.  We  can  project  a  feature  from  the  left  image  to

the  left  image  in  temporal  space,  also  we  can  project  a

feature from the left image to the right image in spatial space.

For different cameras, a different extrinsic transformation Tcb

### should be used.

2)  IMU Factor:  We  use  the  well-known  IMU  preinte- gration algorithm  [11,  12] to construct the IMU factor. We assume that the additive noise in acceleration and gyroscope measurements  are  Gaussian white noise. The time-varying acceleration  and gyroscope bias  are modeled  as  a random walk  process,  whose  derivative  is  Gaussian  white  noise. Since the IMU acquires data at a higher frequency than other sensors, there are usually multiple IMU measurements exist- ing between two frames. Therefore, we pre-integrate IMU measurements on the manifold with covariance propagation. The detailed preintegration can be found at [12]. Within two time instants, t— 1 and t, the preintegration produces relative

position  αtt—1, velocity βtt—1  and rotation 吖tt—1. Also, the

preintegration propagates the covariance of relative position, velocity, and rotation, as well as the covariance of bias. The IMU residual can be deﬁned as:

zimut — himut(X) =

(5)

where  Θ  is  the  minus  operation  on  manifold,  which  is specially used for non-linear rotation. dt is the time interval between two time instants. g is the known gravity vector, whose  norm  is  around  9.81.  Every  two  adjacent  frames

### construct one IMU factor in the cost function.

3)  Other  Factors:  Though we only  specify camera  and IMU factors, our system is not limited to these two sensors. Other  sensors,  such  as  wheel  speedometer,  LiDAR  and Radar, can be added into our  system without much effort. The key is to model these measurements as general residual factors and add these residual factors into cost function.

## C.  Optimization

In traditional, the nonlinear least square problem of eq.3 is solved by Newton-Gaussian or Levenberg-Marquardt ap- proaches. The cost function is linearized with respect to an

initial guess of states, ^X . Then, the cost function is equals

to:

where J is the Jacobian matrix of each factor with respect to

current states ^X . After linearization approximation, this cost

function has closed-form solution of δX. We take Newton- Gaussian as example, the solution is derived as follows,

Finally, current state ^X is updated with ^X    δX, where     is the plus operation on manifold for rotation. This procedure iterates  several  times  until  convergence.  We  adopt  Ceres

solver  [24] to  solve this problem, which utilizes  advanced mathematical  tools  to  get  stable  and  optimal  results  efﬁ- ciently.

## D.  Marginalization

Since  the  number  of  states  increases  along  with  time, the  computational  complexity  will  increase  quadratically accordingly. In order to bound the computational complexity, marginalization is incorporated without loss of useful infor- mation.  Marginalization  procedure  converts  previous  mea- surements into a prior term, which reserves past information. The set of states to be marginalized out is denoted as Xm , and the set of remaining states is denoted as Xr. By summing all marginalized factors (eq.7), we get a new H and b. After rearrange states’ order, we get the following relationship:

The marginalization is carried out using the Schur comple- ment [25] as follows:

(9)

We  get  a  new prior  Hp , bp  for the remaining  states.  The information about marginalized states is converted into prior term without any loss. To be speciﬁc, we keep ten spacial camera frames in our system. When a new keyframe comes, we marginalize out the visual and inertial factors, which are related with states of the ﬁrst frame.

After we  get the prior information  about  current  states, with  Bayes’  rule,  we  could  calculate  the  posterior  as  a product  of  likelihood  and  prior:  p(Xj z)  Ⅸ  p(zjX)p(X). The  state  estimation  then  becomes  a  MAP  (Maximum  A Posteriori) problem. Denote that we keep states from instant m to instant n in the sliding window. The states before m are marginalized out and converted to a prior term. Therefore, the MAP problem is written as:

+ (Hp δXm:n — bp ).

Compared  with  eq.3,  the  above-mentioned  equation  only adds  a  prior  term.  It  is  solved  as  same  as  eq.3  by  Ceres solver [24].

E.  Discussion

The  proposed  system  is  a  general  framework.  Various sensors can be easily added into our system, as long as it can be derived as a general residual factor. Since our system is not specially designed for a certain sensor, it is capable to handle  sensor failure case. When  sensor failure  occurs, we just remove factors of the inactive sensor and add new factors from other alternative sensors.

Translat ion error [m]

Relative Pose Error in MH05

1

0.5

0

2                      10                     20                     30                     40 Distance [m]

Rotation error [deg]

|  |  |  |  | 十 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

P

roposed (stereo)

roposed (mono+imu) roposed (stereo+i

mu)

P

P

4

O

KVIS (stereo+imu

)

3

2

1

0

2                      10                     20                     30                     40 Distance [m]

Fig. 4.     Relative pose error [26] in MH 05 difﬁcult. Two plots are relative errors in translation and rotation respectively.

Translat ion error [m]

Relative Pose Error in V202

0.8

0.6

0.4

0.2

0

2                      10                     20                     30                     40 Distance [m]

| Rotation error [deg] 10 5 0 Fig. 5. | P P P O; roposed (stereo) roposed (mono+i roposed (stereo+ KVIS (stereo+imu); mu) imu). 2 10 20 30 40 Distance [m] Relative pose error [26] in V2 02 medium. Two plots are relative |
| --- | --- |

errors in translation and rotation respectively.

# V.  EXPERIMENTAL RESULTS

We evaluate the proposed system with visual and inertial sensors both  on  datasets  and with real-world  experiments. In the ﬁrst experiment, we compare the proposed algorithm with another state-of-the-art algorithm on public datasets. We then test our system in the large-scale outdoor environment. The numerical analysis is generated to show the accuracy of our system in detail.

## A.  Datasets

We evaluate our proposed system using the EuRoC MAV Visual-Inertial  Datasets  [27].  This  datasets  are  collected onboard a micro aerial vehicle, which contain stereo images (Aptina MT9V034 global shutter, 752x480 monochrome, 20

TABLE I

RMSE[M] IN EUROC DATASET.

| Sequence | Length | Proposed RMSE |  |  | OKVIS RMSE |
| --- | --- | --- | --- | --- | --- |
|  |  | stereo | mono+imu | stereo+imu |  |
| MH 01 | 79.84 | 0.54 | 0.18 | 0.24 | 0.16 |
| MH 02 | 72.75 | 0.46 | 0.09 | 0.18 | 0.22 |
| MH 03 | 130.58 | 0.33 | 0.17 | 0.23 | 0.24 |
| MH 04 | 91.55 | 0.78 | 0.21 | 0.39 | 0.34 |
| MH 05 | 97.32 | 0.50 | 0.25 | 0.19 | 0.47 |
| V1 01 | 58.51 | 0.55 | 0.06 | 0.10 | 0.09 |
| V1 02 | 75.72 | 0.23 | 0.09 | 0.10 | 0.20 |
| V1 03 | 78.77 | x | 0.18 | 0.11 | 0.24 |
| V2 01 | 36.34 | 0.23 | 0.06 | 0.12 | 0.13 |
| V2 02 | 83.01 | 0.20 | 0.11 | 0.10 | 0.16 |
| V2 03 | 85.23 | x | 0.26 | 0.27 | 0.29 |

FPS),  synchronized  IMU  measurements  (ADIS16448,  200 Hz), Also, the ground truth states are provided by VICON and Leica MS50. We run datasets with three different com- binations of sensors, which are stereo cameras, a monocular camera with an IMU, stereo cameras with an IMU separately.	In   this   experiment,   we   compare   our   results    with OKVIS  [8],  a  state-of-the-art  VIO  that  works  with  stereo cameras and an IMU. OKVIS is another optimization-based sliding-window algorithm. OKVIS is specially designed for visual-inertial sensors, while our system is a more general framework, which  supports multiple  sensors combinations. We  tested  the  proposed  framework  and  OKVIS  with  all sequences  in  EuRoC  datasets.  We  evaluated  accuracy  by RPE  (Relative Pose Errors) and ATE  (Absolute Trajectory Errors). The RPE is  calculated by tools proposed in  [26].

The  RPE  (Relative  Pose  Errors)  plot  of  two   sequences, MH 05 difﬁcult and V2 02 medium, are shown in Fig. 4and Fig. 5 respectively.

The  RMSE  (Root  Mean  Square  Errors)  of  ATE  for  all sequences in EuRoC datasets is shown in Table. I. Estimated trajectories  are  aligned  with  the  ground  truth  by  Horn’s method  [28].  The  stereo-only  case  fails  in  V1 03 difﬁcult and V2 03 difﬁcult sequences, where the movement is too aggressive  for  visual  tracking  to  survive.  Methods  which involves  the  IMU  work  successfully  in  all  sequences.  It is  a  good  case  to   show  that  the  IMU  can  dramatically improve motion tracking performance by bridging the gap when visual tracks fail due to illumination change, texture- less area, or motion blur.

From the relative pose error and absolute trajectory error, we can see that the stereo-only method performed worst in most sequences. Position and rotation drift obviously grown along  with  distance  in  stereo-only  case.  In  other  words, the IMU signiﬁcantly beneﬁted vision in states estimation. Since  the  IMU  measures  gravity  vector,  it  can  effectively suppress drifts in roll and pitch angles. Stereo cameras with an IMU didn’t always perform best, because it requires more accurate calibration than the case of a monocular camera with an  IMU.  Inaccurate  intrinsic  and  extrinsic  calibration  will introduce more noise into the  system. In general, multiple sensor  fusion  increase  the  robustness  of  the  system.  Our results outperforms OKVIS in most sequences.

Fig. 6.     The self-developed sensor suite used in the outdoor environment. It  contains  stereo  cameras  (mvBlueFOX-MLC200w,  20Hz)  and  DJI  A3 controller, which include inbuilt IMU (200Hz) and GPS receiver.

Outdoor Trajecotry

20

Proposed(stereo)

Proposed(mono+imu)

15

Proposed(stereo+imu) GPS

10

y [m]

5

0

-5

-10

-15

-20

-5               0                5               10              15              20              25              30

x [m]

Fig. 7.     Estimated trajectories in outdoor experiment.

## B.  Real-world experiment

In this experiment, we used a self-developed sensor suite to demonstrate our framework. The sensor suite is shown in Fig. 6. It contains stereo cameras (mvBlueFOX-MLC200w, 20Hz) and DJI A3 controller2, which inculdes inbuilt IMU (200Hz) and GPS receiver. The GPS position is treated as ground truth. We hold the  sensor  suite by hand  and walk

2http://www.dji.com/a3

TABLE II

RMSE[M] IN OUTDOOR EXPERIMENT.

| Sequence | Length | Proposed RMSE |  |  |
| --- | --- | --- | --- | --- |
|  |  | stereo | mono+imu | stereo+imu |
| outdoor1 | 223.70 | 1.85 | 0.71 | 0.52 |
| outdoor2 | 229.91 | 2.35 | 0.56 | 0.43 |
| outdoor3 | 232.13 | 2.59 | 0.65 | 0.75 |

Relative Pose Error in Outdoor Dataset

mono+imu

|  | stereo |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | stereo+im |  | u |  |  |  |  |  |  |  |  |  |  |

Translat ion error [m]

3

2.5

2

1.5

1

0.5

0

2                      20                     30                     40                     50

Distance [m]

Fig. 8.      Relative pose error [26] in outdoor experiment.

around on the outdoor ground. We run states estimation with three  different  combinations,  which  are  stereo  cameras,  a monocular camera with an IMU, and stereo cameras with an IMU.

For accuracy comparison, we walked two circles on the ground and compared our estimation with GPS. The trajec- tory is shown in Fig. 7, and the RPE (Relative Pose Error) is shown in Fig. 8. As same as dataset experiment, noticeable position drifts occurred in the stereo-only scenario. With the assistance  of  the  IMU,  the  accuracy  improves  a  lot.  The RMSE of more outdoor experiments is shown in Table. II. The method which involves the IMU always performs better than the stereo-only case.

# VI.  CONCLUSION

In this paper we have presented a general optimization- based  framework  for  local  pose  estimation.  The  proposed framework can support multiple sensor combinations, which is  desirable  in  aspect  of robustness  and  practicability.  We further demonstrate it with visual and inertial sensors, which form three sensor suites (stereo cameras, a monocular camera with  an  IMU,   and  stereo  cameras  with   an  IMU).  Note that although we only show the factor formulations for the camera  and  IMU,  our  framework  can  be  generalized  to other  sensors as well. We validate the performance of our system  with  multiple  sensors  on  both  public  datasets  and real-world experiments. The numerical result indicates that our  framework  is  able  to  fuse  sensor  data  with  different settings.

In future work, we will extend our framework with global sensors (e.g. GPS) to achieve locally accurate and globally aware pose estimation.

# REFERENCES

[1]  G. Klein and D. Murray, “Parallel tracking and mapping for small ar workspaces,” in Mixed and Augmented Reality, 2007. IEEE and ACM International Symposium on, 2007, pp. 225–234.

[2]  C. Forster, M. Pizzoli,  and D.  Scaramuzza,  “SVO: Fast  semi-direct monocular visual odometry,” in Proc. of the IEEE Int. Conf. on Robot. and Autom., Hong Kong, China, May 2014.

[3]  J.  Engel,  T.   Scho… ps,  and  D.  Cremers,   “Lsd-slam:  Large-scale  di- rect monocular slam,” in  European  Conference  on Computer Vision. Springer International Publishing, 2014, pp. 834–849.

[4]  R. Mur-Artal, J. Montiel, and J. D. Tardos, “Orb-slam: a versatile and accurate monocular slam system,” IEEE Trans. Robot., vol. 31, no. 5, pp. 1147–1163, 2015.

[5]  J. Engel, V. Koltun, and D. Cremers, “Direct sparse odometry,” IEEE Transactions on Pattern Analysis and Machine Intelligence, 2017.

[6]  A. I. Mourikis and S. I. Roumeliotis, “A multi-state constraint Kalman ﬁlter for vision-aided inertial navigation,” in Proc. of the IEEE Int. Conf. on Robot. and Autom., Roma, Italy, Apr. 2007, pp. 3565–3572.

[7]  M. Li and A. Mourikis, “High-precision, consistent EKF-based visual- inertial odometry,” Int. J. Robot. Research, vol. 32, no. 6, pp. 690–711, May 2013.

[8]  S.  Leutenegger,  S.  Lynen,  M.  Bosse,  R.  Siegwart,  and  P.  Furgale, “Keyframe-based visual-inertial  odometry using nonlinear  optimiza- tion,” Int. J. Robot. Research, vol. 34, no. 3, pp. 314–334, Mar. 2014.

[9]  M. Bloesch,  S.  Omari, M. Hutter,  and R.  Siegwart,  “Robust visual inertial odometry using a direct ekf-based approach,” in Proc. of the IEEE/RSJ Int.  Conf.  on Intell. Robots  and Syst.     IEEE,  2015,  pp. 298–304.

[10]  R. Mur-Artal and J. D. Tards, “Visual-inertial monocular slam with map reuse,”  IEEE Robotics and Automation Letters, vol. 2, no. 2, pp. 796–803, 2017.

[11]  C. Forster, L. Carlone, F. Dellaert, and D. Scaramuzza, “On-manifold preintegration  for  real-time  visual-inertial  odometry,”  IEEE  Trans. Robot., vol. 33, no. 1, pp. 1–21, 2017.

[12]  T.  Qin,  P.  Li,  and   S.  Shen,  “Vins-mono:  A  robust   and  versatile monocular visual-inertial state estimator,” IEEE Trans. Robot., vol. 34, no. 4, pp. 1004–1020, 2018.

[13]  J. Zhang and S. Singh, “Loam: Lidar odometry and mapping in real- time.” in Robotics: Science and Systems, vol. 2, 2014, p. 9.

[14]  C.  Kerl,  J.  Sturm,  and  D.  Cremers,  “Dense  visual  slam  for  rgb-d cameras,” in Proc. of the IEEE/RSJ Int. Conf. on Intell. Robots and Syst.

[15]  H. Rebecq, T. Horstschaefer, G. Gallego, and D. Scaramuzza, “Evo: A geometric approach to event-based 6-dof parallel tracking and mapping in real time,”  IEEE Robotics  and Automation Letters,  vol. 2, no. 2, pp. 593–600, 2017.

[16]  S.  Lynen,  M.  W.  Achtelik,   S.  Weiss,  M.  Chli,  and  R.   Siegwart, “A robust and modular multi-sensor fusion approach applied to mav navigation,” in Proc. of the IEEE/RSJ Int. Conf. on Intell. Robots and Syst.    IEEE, 2013, pp. 3923–3929.

[17]  G. P. Huang, A. I. Mourikis, and S. I. Roumeliotis, “Observability- based rules for designing consistent ekf slam estimators,” Int. J. Robot. Research, vol. 29, no. 5, pp. 502–528, 2010.

[18]  S. Shen, Y. Mulgaonkar, N. Michael, and V. Kumar, “Multi-sensor fu- sion for robust autonomous ﬂight in indoor and outdoor environments with a rotorcraft MAV,” in Proc. of the IEEE Int. Conf. on Robot. and Autom., Hong Kong, China, May 2014, pp. 4974–4981.

[19]  R. Ku… mmerle, G. Grisetti, H. Strasdat, K. Konolige, and W. Burgard, “g2o: A general framework for graph optimization,” in Proc. of the IEEE Int. Conf. on Robot. and Autom.    IEEE, 2011, pp. 3607–3613.

[20]  M.  Kaess,  H.  Johannsson,  R.  Roberts,  V.  Ila,  J.  J.  Leonard,  and F.  Dellaert,  “isam2:  Incremental  smoothing  and  mapping  using  the bayes tree,” Int. J. Robot. Research, vol. 31, no. 2, pp. 216–235, 2012.

[21]  H. Liu, M. Chen, G. Zhang, H. Bao, and Y. Bao, “Ice-ba: Incremental, consistent and efﬁcient bundle adjustment for visual-inertial slam,” in Proc. of the IEEE Int. Conf. on Pattern Recognition, 2018, pp. 1974– 1982.

[22]  J. Shi and C. Tomasi, “Good features to track,” in  Computer Vision and Pattern Recognition,1994. IEEE Computer Society Conference on, 1994, pp. 593–600.

[23]  B. D. Lucas and T. Kanade, “An iterative image registration technique with an application to stereo vision,” in Proc. of the Intl. Joint Conf. on Artiﬁcial Intelligence, Vancouver, Canada, Aug.  1981, pp. 24–28.

[24]  S. Agarwal, K. Mierle, and Others, “Ceres solver,” http://ceres-solver. org.

[25]  G. Sibley, L. Matthies, and G. Sukhatme, “Sliding window ﬁlter with application to planetary landing,” J. Field Robot., vol. 27, no. 5, pp. 587–608, Sep. 2010.

[26]  A. Geiger, P. Lenz, and R. Urtasun, “Are we ready for autonomous driving? the kitti vision benchmark suite,” in Proc. of the IEEE Int. Conf. on Pattern Recognition, 2012, pp. 3354–3361.

[27]  M. Burri, J. Nikolic, P. Gohl, T. Schneider, J. Rehder, S. Omari, M. W. Achtelik, and R. Siegwart, “The euroc micro aerial vehicle datasets,” Int. J. Robot. Research, 2016.

[28]  B. K. Horn, “Closed-form solution of absolute orientation using unit quaternions,” JOSA A, vol. 4, no. 4, pp. 629–642,  1987.